# Multi-Agency Drone/Sensor COP – Technical Standards

## 1. Introduction and Architecture

A multi-agency Drone Common Operating Picture (COP) requires a unified situational awareness built on open, modular principles. We adopt a Modular Open Systems Approach (MOSA) and Sensor Open Systems Architecture (SOSA) mindset: interfaces are standardized, the core system uses only open formats, and new sensors plug in via “adapters” that normalize proprietary data. This avoids vendor lock-in and ensures the core COP can scale and evolve without redesign. Each sensor (radar, camera, RF, etc.) outputs in a canonical format (or an adapter converts it) so the COP’s internal “bus” carries only open, interoperable protocols. The COP then fuses and displays all tracks in real time, with role-based access so agencies see the data they’re cleared to see (hiding any sensitive details).

Each chapter below details the recommended standards and why they were chosen, with alternatives noted. A summary at the end of each chapter highlights key points.

### 1.1 Core Objectives

- Unified COP – All sensors feed a common display of tracks/alerts, eliminating silos.

- Modular Integration – New sensor types connect via standardized adapters, without re-architecting the system.

- Multi-Agency Roles – Fine-grained access (Attribute-Based Access Control) ensures data visibility only to authorized users, even across agencies.

- Real-Time Performance – Sensors timestamped with microsecond accuracy and prioritized on the network so detection data shows up with minimal delay.

- Audit and Provenance – Every data packet and user action is logged in structured, tamper-resistant logs for accountability.

Summary: The system is designed as an open, modular COP: plug-and-play sensors, standardized data models, and secure sharing across agencies. This maximizes interoperability and future-proofs the architecture.

## 2. Network Infrastructure Standards

### 2.1 IP Networking and Segmentation

We build on an IPv4/IPv6 dual-stack network, since IPv6 provides virtually unlimited addresses for many sensors, while maintaining IPv4 compatibility. All new hardware must support both protocols. Sensors and systems are grouped into VLANs by function (e.g. radar VLAN, camera VLAN, management VLAN) and by trust level. No implicit routing is allowed between VLANs – traffic is explicitly filtered by firewalls. This segmentation contains any breaches and limits broadcast domains, improving both security and performance.

Why IPv6? Large-scale sensor deployments (airports, borders, cities) require many IPs, which IPv6 easily supplies. IPv4 runs in parallel for legacy gear. VLANs create “air-gaps” logically: e.g. a radar VLAN can’t directly talk to a camera VLAN unless rules allow it.

### 2.2 Multicast and Data Distribution

#### Primary: Source-Specific Multicast (SSM)

For efficient one-to-many data streams (like video and radar feeds), we use Source-Specific Multicast (SSM) via PIM-SSM routing. In SSM, each multicast “channel” is defined by a specific (S,G) pair (source IP, group IP)[1]. Receivers explicitly join the stream from a trusted source S, so they cannot be tricked into receiving data from unknown sources. In effect, receivers say “only take traffic from this source IP for this group IP,” which eliminates the ASM risk of rogue data injection[1]. The reserved IPv4 SSM range (232.0.0.0/8) and IPv6 SSM range (FF3x::/32) are used. Receivers must run IGMPv3 (IPv4) or MLDv2 (IPv6) to filter by source.

- Rationale: SSM’s (S,G) model gives each agency confidence they receive only authenticated sensor streams. Unlike Any-Source Multicast (ASM), unauthorized sources cannot inject fake video or radar data, since no one will join the (S,G) if S is unknown or malicious[1].

- Implementation: Network devices (switches/routers) must support PIM-SSM and IGMPv3/MLDv2 snooping. Multicast from legacy systems (ASM) is avoided or given special exception.

Alternative (unicast): If multicast isn’t possible (e.g. public networks), a brokered approach (MQTT, AMQP, WebSockets) can deliver the data, though this adds latency and bandwidth overhead.

#### 2.3 Quality of Service (QoS) and TSN

Sensor data have different urgency. We mark traffic with Differentiated Services Code Points (DSCP) to prioritize it in the network[2]. For example, critical control messages (flight safety alerts) get Expedited Forwarding (EF, DSCP 46) for lowest latency, while video streams use Assured Forwarding (AF41, DSCP 34). Even within video/radar, mission-critical tracks might use higher-priority AF41, while routine feeds might use AF31 (DSCP 26). By classifying traffic, switches can schedule queues so a big video upload can’t starve out a high-priority alert.

- TSN Scheduling: On TSN-capable switches, 802.1Qbv Time-Aware Shaping enforces fixed transmission windows for high-priority traffic, giving deterministic latency. For example, a 4K camera’s bulk stream is held back when an alert packet’s slot comes up.

- Mapping: All devices mark packets at source. Switches and routers have consistent queue-maps (e.g. DSCP 46→priority 1 queue). This ensures end-to-end QoS.

- Rationale: Without QoS, a sudden flood of video could delay urgent track data. DSCP+TSN ensures “Drone Detected!” alerts cut through any congestion.

Summary: Multicast (SSM) efficiently distributes sensor streams, and QoS/TSN ensures real-time data (alerts, tracking) always get priority over best-effort traffic[1][2].

## 3. Timing and Synchronization

Accurate time is the glue that fuses multi-sensor data. We use IEEE 1588-2019 Precision Time Protocol (PTP) with boundary clocks for sub-microsecond synchronization. The PTP grandmaster (GPS-referenced) propagates time to switches and devices; boundary clocks in switches regenerate the signal across segments, avoiding jitter buildup. The goal is ≤1 μs end-to-end skew within the local campus. High-stability oscillators (OCXOs) on grandmasters, and support for Transparent/Boundary Clock modes in intermediate switches, are required.

- Why PTP? PTP achieves sub-microsecond accuracy on LANs[2], far better than NTP. This precision lets the COP know, for example, that Radar A saw the drone at 12:34:56.001234 UTC and RF Sensor B saw it at 12:34:56.001789 – clearly the same event. Without PTP, loose timestamps could make correlation unreliable.

- GPSTie-in: Grandmasters are GPS-disciplined. Redundancy is required (at least two masters with failover via Best Master Clock Algorithm).

- Boundary Clocks: Every switch between grandmaster and sensors must either pass PTP timestamps (transparent clock) or act as a boundary clock, so hop delays are compensated. This keeps synchronization tight.

- Reference: The IEEE 1588 standard itself (Precision Time Protocol) defines this model[2].

### 3.1 PTP Security

Time is a high-value target: a false master could inject bad time and scramble the COP. We implement the new IEEE 1588-2023 security extensions (often called “Prong A–D” security) as a defense-in-depth:

- Prong A – Cryptographic Authentication: All PTP packets carry Authentication TLVs (e.g. HMAC) so slaves verify messages came from an authorized master.

- Prong B – Transport Security: PTP traffic is on VLANs protected by MACsec (IEEE 802.1AE) encryption[3]. This prevents eavesdropping or injection at the link layer.

- Prong C – Topology Checks: Switches filter PTP BMC messages so only expected master ports are allowed. Unusual path changes trigger alarms.

- Prong D – Monitoring: Automated monitors watch for anomalies (sudden offset jumps, repeated BMCA elections, etc.) and alert operators to possible time attacks.

Why MACsec? MACsec provides hardware-based line-rate encryption and integrity (often AES-GCM-256)[4], so PTP messages can’t be tampered without detection and are isolated from intruders. It works on Ethernet so it protects broadcast/multicast as well – crucial since PTP uses Ethernet frames.

(Note: Emerging NTS4PTP (network time security) is not yet mature for production.)

### 3.2 Alternative: gPTP/802.1AS for TSN

In fully TSN networks, IEEE 802.1AS (gPTP) can be used for time sync. It’s essentially PTP tailored to Ethernet AVB/TSN, providing nanosecond-level sync in switches optimized for TSN. Use gPTP on segments where ultra-precise alignment (nanoseconds) is needed for scheduled traffic, e.g. aligning video frames with RF timestamps[5]. Otherwise, standard IEEE 1588 suffices.

Summary: We synchronize all devices to the microsecond level using IEEE 1588 PTP with secure boundaries. This lets every sensor’s data be accurately time-tagged for fusion. All PTP traffic is authenticated and encrypted (MACsec) to prevent time spoofing. Sub-nanosecond gPTP (802.1AS) can be used in TSN segments if needed[2][3].

## 4. Security Standards

### 4.1 Network Admission (802.1X & DevID)

To enforce a zero-trust network, IEEE 802.1X port-based NAC is mandatory on all Ethernet ports serving sensors or COP devices. Each device must present a hardware-backed certificate (IEEE 802.1AR DevID) during login. Switch ports only allow traffic from authenticated devices, and unknown devices go into a quarantine VLAN.

- Why DevID? The 802.1AR Secure Device Identity (DevID) standard ensures each device has a unique cryptographic key burnt into it[3]. This binds the certificate to the hardware, making it hard to spoof a device’s identity.

- Operation: On link-up, the sensor uses EAP-TLS (or EAP-TLSv2) to authenticate to an 802.1X server. The server verifies the DevID certificate (from a trusted CA). If authentication fails, the port shuts down or restricts to management network.

- Logging: Every auth attempt (success or fail) is logged for audit.

Summary: Only authorized, certified devices can join the network. 802.1X/EAP-TLS with IEEE 802.1AR DevID certificates prevents a stolen sensor (or rogue laptop) from joining undetected[3].

### 4.2 Link-Layer Encryption (MACsec)

All switch-to-switch and switch-to-server links in critical segments must use IEEE 802.1AE MACsec. This encrypts every Ethernet frame between peers, including multicast and broadcast, at line rate. We require AES-GCM-256 cipher suites with rotating keys (every ≤24h). Separate MACsec key chains are used for data vs. management VLANs.

- Why MACsec? Unlike IPsec or TLS, MACsec is done in hardware on the wire with negligible latency impact[4]. It protects against eavesdropping and frame injection across the LAN. (It doesn’t protect against rogue devices already inside the encrypted LAN, which is why we also use 802.1X NAC.)

- Implementation: Switches must support MACsec (many enterprise/industrial switches do). Each link uses MKA (MACsec Key Agreement) to exchange keys automatically using the DevID credentials.

- Management: Network management traffic (SNMP, SSH, etc.) also goes over MACsec to protect it.

Summary: Every hop of sensor data is encrypted by MACsec at layer 2, providing confidentiality and integrity without impacting throughput[4].

### 4.3 Logging and Audit

All components log to a central, secure syslog server using RFC 5424 format over TLS (RFC 5425). Logs include date/time, sensor IDs, user IDs, event types, and must survive reboots (write-once storage).

- Structure: We use structured (CAMP) syslog so automation can parse fields (e.g. sensorType, severity, messageID).

- Transport: Syslog over TLS (port 6514) with mutual certificate auth prevents tampering in transit. We use TLS 1.3 ciphers (no old CBC).

- Retention: Audit trails are stored in append-only logs (WORM storage) to be tamper-evident.

- Examples: Login events, configuration changes, data publication records, and security alerts all produce log entries.

Summary: Every action is logged in a standardized, encrypted way. This ensures full auditability of data and access, critical for forensics and compliance.

## 5. Sensor Interface Standards

All sensor “plugins” (adapters) output data in the following open formats and protocols. The core COP ingests these standardized streams:

### 5.1 Radar and MTI

- Track Data: Use EUROCONTROL ASTERIX CAT-062 (System Track Messages) for moving target reports[6]. This binary format includes position, velocity, track ID, time-of-track, and sensor codes. It is compact and broadly supported by radar vendors and C2 systems worldwide. Mandatory fields include System ID (SAC/SIC), Track ID, timestamp, lat/long/altitude, and status. (If IP fragmentation might occur, ensure messages fit the MTU.)

Alternative: NATO’s SAPIENT format (used by UK C-UAS systems) is another option, especially for feeding high-level alarms. SAPIENT adds confidence and category fields. A practical architecture might carry both: ASTERIX for raw tracks and SAPIENT only for fused alerts.

- Radar Video: Use ASTERIX CAT-240 for live radar video dumps. It standardizes packaging of rotating radar video sweeps over IP. This allows any compliant display to show vendor radar imagery. Transport CAT-240 as UDP/SSM to users. Frame rates should sync to PTP timebase for consistency (each frame stamped with PTP time).

Example Reference: ASTERIX CAT-062 is defined by Eurocontrol as the standard exchange for System Track Data[6]. It ensures different radars and C2 systems speak the same “track language”.

### 5.2 RF and SDR

- Live RF Data: Use ANSI/VITA 49.2 (VRT) for real-time spectral data. VRT packets carry streaming I/Q samples along with timestamps and context (center frequency, sample rate, gain)[7]. PTP-synchronized timestamps on each packet allow signals from distributed receivers to be correlated.

Archive: Store captured RF data in SigMF format. SigMF files contain raw binary I/Q data plus a JSON metadata file describing center freq, capture time, etc. SigMF’s open JSON schema makes long-term archival and analysis easy[8].

Why VITA-49 / SigMF? VITA-49 makes SDR hardware interoperable and time-aligned[7], and SigMF avoids proprietary trap by separating data (binary) from metadata (JSON)[8].

### 5.3 EO/IR Video (Full-Motion Video)

- Standard: STANAG 4609 (NATO Digital Motion Imagery) with MISB KLV metadata. The video stream is H.264/AVC or H.265 inside MPEG-TS, with embedded KLV data in MISB format. This carries precise geo-tags: platform GPS (MISB ST 0601), accurate timestamps (ST 0603/0604), camera orientation (in ST 0601), and optional moving target cues (MISB ST 0903 VMTI).

- Synchronization: Video frames and metadata are synchronized via PTP (SMPTE ST 2059-2 profile) so each metadata packet has an accurate timestamp relative to the video. If using MPEG-TS, insert KLV either synchronously (PTS-aligned) or asynchronously per MISB guidance.

- Transport: Locally, use UDP/SSM for multicast FMV feeds. Over WAN, use RIST (VSF TR-06) or SRT to handle packet loss and encryption. Both support ARQ (retransmit missing packets) for reliability and DTLS/AES for security. RIST Main Profile is preferred for multicast/bonding (e.g. dual 4G links), while SRT is fine for point-to-point links.

Standards Reference: STANAG 4609 (2023) is the NATO standard for interoperable motion video, relying on MISB KLV sets[9][10]. It ensures any screen can overlay a camera “cone” on the map by reading the geo-metadata.

### 5.4 Acoustic Sensors

- Audio streams: Use RTP with AES67 for microphone arrays or acoustic sensors. AES67 is the industry standard for synchronized, low-latency audio over IP. It uses multicast and is PTP-synced to ensure all sound channels align in time (so you can do beamforming or triangulation).

### 5.5 UAS Remote ID

- Broadcast Messages: Parse any ASTM F3411-22a Remote ID broadcasts (Bluetooth/Wi-Fi). That is the global aviation standard for drones to announce ID, position, etc.

- DRIP Trust: Implement IETF DRIP (RFC 9434) for cryptographic authentication of Remote ID.[11]. In practice, verify DRIP signatures: each broadcast ID packet contains a Host Identity Tag (HIT) which is signed by a public key. We check that against known registries (IANA/DRIP) to see if the drone is “blue” (registered friendly) or “red” (fake). If the signature fails or is unknown, we flag the drone as untrusted.

Why DRIP? Without DRIP, a hostile actor could spoof a friendly drone’s ID. The DRIP framework provides a way to trust a Remote ID by chaining it to a PKI/RPKI infrastructure[11]. This is crucial for law enforcement vs. hostile detection.

Summary: Every sensor’s raw output is translated into an open, documented format (ASTERIX, VITA-49, MISB, etc.), complete with PTP timestamps. This lets the COP ingest disparate data seamlessly. Metadata (especially geo-coordinates and track quality) travels with the data for the fusion engine to use.

## 6. Data Fusion and Common Operating Picture

### 6.1 Track Exchange and Fusion

We use STANAG 4676 as the canonical track exchange format. STANAG 4676 is a NATO ISR standard for exchanging track data[12]. It includes not just position/velocity/time, but also covariance matrices, track history, and data provenance. Each sensor feed provides STANAG 4676-style track messages (e.g. via DDS or a REST API). The central fusion engine merges tracks from multiple sensors into unified “fused tracks,” updating confidence and lineage.

- Provenance: Each track carries a sensor ID, confidence score, and timestamp. When tracks are fused or split, the system records the lineage (e.g. “Track 1234 was joined from Radar A and EO Camera B”).

- Coast Guard & Allies: Because STANAG 4676 is widely adopted among NATO and industry fusion tools, it ensures allied forces and different vendors speak the same track language[12].

- Alternative: If STANAG 4676 isn’t available, one could use a custom JSON schema or a DDS topic (e.g. JAUS IFF messages) as long as it carries the same info, but STANAG 4676 is preferred for multi-vendor consistency.

### 6.2 Data Access and Labeling

We enforce Attribute-Based Access Control (ABAC) with data labeling. Every data element (track, video frame, log) carries a sensitivity tag. For example, a military radar track might be labeled “NATO-SECRET”. When sharing data between agencies, the COP applies filtering: e.g., for a border-patrol user, a secret label may be downgraded or obfuscated.

- NATO Classification: We use the NATO STANAG 4774 labeling syntax[13] to tag data. A STANAG-4774 label includes fields like classification level, source country, originator, and release marking.

- Label Binding: As per STANAG 4778, each data packet’s label is cryptographically bound so it can’t be removed without detection[13].

- “Downgrading” Example: If a Track from a “SECRET” sensor must be shared with “NATO-RESTRICTED” partners, the COP might send only the track coordinates (not source or altitude) or shift them slightly, according to policy. All such access decisions are logged.

Why ABAC? Role-based (RBAC) is too coarse for multi-national data sharing. With ABAC, policies can say things like “if user.country != track.origin_country, then mask the altitude.” The STANAG 4774/4778 framework provides a proven way to label and enforce these rules in coalition environments[13].

Summary: Track data and video streams are fused into unified tracks using the STANAG 4676 data model[12]. Every piece of data is tagged with a security label (STANAG 4774 format) and ABAC policies control who sees what, with cryptographic binding to prevent tampering[13].

## 7. Backhaul and Resilience

### 7.1 WAN Streaming (4G/5G/Internet Links)

For linking remote sites (e.g., towers, mobile units) over lossy networks, use RIST or SRT for video and critical data streams. Both protocols sit above UDP and add reliability via retransmission (ARQ), with low latency:

- RIST (Main Profile): Preferred for multicast-like distribution and bonding multiple links. RIST automatically retransmits lost RTP packets and can bond several internet links (4G + fiber) for higher throughput. It also supports out-of-band management and DTLS encryption[14].

- SRT: Good for point-to-point feeds. SRT uses a UDP-based “secure” protocol with packet recovery and AES encryption. It maintains ~120ms latency while repairing losses[15].

- Implementation: Video encoders (Haivision, Matrox, etc.) should support RIST/SRT. Backhaul gear must monitor link health and switch paths if a link dies (target <500ms switchover).

- Protocol Choice: If distributing to many recipients, RIST (with multicast PR (Playout Recovery) support) is handy. If it’s a dedicated link (e.g. tower→central), SRT works well.

### 7.2 Redundancy (802.1CB FRER)

To guarantee zero loss on critical LAN links, we use IEEE 802.1CB Frame Replication and Elimination for Reliability. FRER automatically sends duplicate frames over disjoint paths: if one path loses a packet, the duplicate on the other path arrives. The receivers drop duplicate copies based on sequence numbers.

- Where: Implement FRER at the network edge, such as between two switches or along separate routes to the same destination.

- Why: This delivers “zero loss” for essential streams (radar video, tracks) without waiting for retransmit (which could be too late).

- Example: A radar’s video feed is sent over two redundant VLANs to the central display. The switch core merges frames and drops dupes[16]. If one switch fails, the other takes over seamlessly.

- Monitoring: The system tracks “Replication Efficiency” (how often duplicates are used) and path health.

Summary: For unreliable links (WAN or even campus backplanes), RIST/SRT provides reliable video streaming with ARQ[14][15]. For LAN fault tolerance, IEEE 802.1CB (FRER) duplicates critical frames across paths so no packet is lost[16].

## 8. Configuration and Telemetry

### 8.1 Configuration Management

We use NETCONF (RFC 6241) with YANG models for all device configurations. NETCONF provides a secure, transactional way to push configs (and roll them back if errors occur)[17]. Each device (switch, server, sensor) exposes a YANG-based data model for its settings (IP addresses, VLANs, keying, etc.).

- Why NETCONF/YANG? Unlike CLI scripts, YANG models give a clear schema and validation. You can audit and version-control configs easily. RESTCONF (RFC 8040) is an alternative if pure HTTP/REST is preferred, but still based on YANG.

- Usage: A centralized orchestrator (or certified config tool) pushes configs via NETCONF. Eg. VLAN assignments, QoS maps, PTP profiles are all set via standardized models.

- Rollback: If a bad change is detected, NETCONF’s <confirm-commit> or rollback mechanisms restore the previous config automatically.

### 8.2 Telemetry Streaming

Instead of polling SNMP, use gRPC Network Management Interface (gNMI) with OpenConfig YANGs for telemetry. gNMI allows high-frequency, push-based subscriptions for metrics (link stats, CPU, PTP offsets, etc.).

- On-Change Subscriptions: Critical parameters (GPS lock, alarm flags, PTP state changes) are sent immediately when they change.

- Performance: gNMI over HTTP/2 is much lighter-weight and higher rate than SNMP. Good for hundreds of metrics per second if needed.

- Security: gNMI uses TLS with mutual cert auth for encryption and identity.

Summary: Devices are managed via NETCONF/YANG (or RESTCONF) for structured, auditable config[17]. Telemetry flows via gNMI streams so operators see real-time health and state.

## 9. Testing and Conformance

We will test each segment against defined criteria:

- Timing (PTP): Measure end-to-end clock offset under load; must stay ≤1 μs. Test BMCA failover time (aim <1 sec). Verify PTP auth rejects spoofed messages.

- Multicast: Verify IGMPv3/MLDv2 membership changes propagate (<1 sec). Test SSM joins only accept authorized sources. Ensure FRER recovers without loss on single-link failure[16].

- Security: Follow NIST SP 800-115 style pentests. Probe 802.1X bypass, attempt MACsec tampering (should be prevented). Test PTP security (prongs A–D) under attack scenarios. Confirm audit logs are complete after reboots.

Summary: Use standardized tests (RFC or industry tools) to confirm timing, multicast, and security meet the requirements. Any failures trigger fixes before going live.

## 10. Summary of Key Points

- Open, Interoperable Design: The COP uses only open standards (ASTRIX, STANAG, VITA-49, AES67, etc.) for all sensor data, so multiple vendors can plug in easily.

- Secure, Segmented Network: Dual-stack IPv4/IPv6, VLAN segmentation, 802.1X devID, and MACsec provide a zero-trust network foundation[3][4].

- Reliable, Low-Latency Transport: PIM-SSM multicast for distribution, DSCP/TSN for prioritization, and RIST/SRT for WAN keep data flowing with minimal delay. FRER ensures no packets are lost in the LAN[1][14].

- High-Accuracy Time: IEEE 1588 PTP (or gPTP) locks all clocks to microsecond/nanosecond precision, so data from different sensors can be fused correctly[2].

- Fusion-Friendly Formats: Radar → ASTERIX-62/240, EO video → STANAG-4609/MISB, RF → VITA-49/SigMF, etc. These standards encapsulate all needed info (position, time, context).

- Trusted Remote ID: ASTM F3411 for format, plus IETF DRIP for cryptographic validation, ensures we can tell genuine drones from spoofers[11].

- Access Control: Data is labeled with STANAG 4774 metadata and ABAC rules enforce who sees what, supporting multi-national scenarios[13].

- Management: Configuration and telemetry use modern network-management protocols (NETCONF/gNMI), allowing automated control and monitoring.

By adhering to these standards and rationale, the resulting COP system will be modular, secure, and robust, giving all agencies a shared, real-time view of the drone/sUAS threat environment.

Sources: Authoritative standards and technical literature as cited above (e.g. IEEE 1588 PTP[2], ASTERIX[6], VITA 49[7], MISB/STANAG-4609[9][10], NATO STANAG 4676[12], RIST/SRT protocols[14][15], and NATO STANAG 4774/4778 labeling[13]).

[1]  RFC 3569 - An Overview of Source-Specific Multicast (SSM)

https://datatracker.ietf.org/doc/html/rfc3569

[2] [5] Precision Time Protocol - Wikipedia

https://en.wikipedia.org/wiki/Precision_Time_Protocol

[3] IEEE 802.1X - Wikipedia

https://en.wikipedia.org/wiki/IEEE_802.1X

[4] What is MACsec? - Comcores

https://www.comcores.com/what-is-macsec/

[6] cat062p9ed110

https://www.eurocontrol.int/sites/default/files/content/documents/nm/asterix/archives/asterix-cat062-system-track-data-part9-v1.10-122009.pdf

[7] VITA - VITA 49 Overview

https://www.vita.com/page-1855484

[8] sigmf.org

https://sigmf.org/sigmf-spec.pdf

[9] [10] STANAG 4609 – ISR Video – ImpleoTV

https://impleotv.com/2025/03/11/stanag-4609-isr-video/

[11]  Drone Remote ID Protocol (drip)

https://datatracker.ietf.org/group/drip/about/

[12] api.army.mil

https://api.army.mil/e2/c/downloads/2023/01/31/daf24e67/nato-standardization-agreements-apr-19-public.pdf

[13] Ensuring NATO STANAG 4774 and 4778 Compliance

https://www.archtis.com/achieving-nato-stanag-4774-and-4778-compliance/

[14] Reliable Internet Stream Transport - Wikipedia

https://en.wikipedia.org/wiki/Reliable_Internet_Stream_Transport

[15] Secure Reliable Transport - Wikipedia

https://en.wikipedia.org/wiki/Secure_Reliable_Transport

[16] P802.1CB – Frame Replication and Elimination for Reliability |

https://1.ieee802.org/tsn/802-1cb/

[17]  RFC 6241 - Network Configuration Protocol (NETCONF)

https://datatracker.ietf.org/doc/html/rfc6241