find more examples where counter-uav solutions vendors have already used it to lock in. find examples with quotes that I could refer to. (very good example is Denmark project mentioned PREVIOUSLY) . if possible, find a standard or protocol that caused this warning! (focus on ASTERIX, SAPIENT, MOSA, SOSA). try to find more recent. examples from law enforcement or aviation.

This report details examples of vendor lock-in in the Counter-UAS (C-UAS) market, contrasting them with the "Denmark Project" (DALO/Terma) mentioned previously. It also identifies the specific protocols and standards central to this issue.

1. The Protocol That "Caused the Warning"

The "warning" issued by bodies like NATO and the UK Centre for the Protection of National Infrastructure (CPNI/NPSA) was largely triggered by the widespread reliance on Proprietary Interface Control Documents (ICDs) and the misuse of the ASTERIX standard.

While ASTERIX (All Purpose Structured Eurocontrol Surveillance Information Exchange) is an open standard, its limitations for drone warfare created a loophole that vendors exploited to create lock-in.

- The Trap (ASTERIX Cat 048 Misuse): The traditional standard for radars is ASTERIX Category 048 (Monoradar Target Reports). However, Cat 048 was designed for airplanes, not drones. It lacks standard fields for critical drone data like "Drone ID," "RF Signature," or "Confidence Level."

- How Lock-In Happens: To transmit this extra data, vendors used "User Defined" or "Reserved" fields within the ASTERIX message. Since only their own Command & Control (C2) software knew how to decode these proprietary extensions, the customer was locked into buying the same vendor's software forever. If they switched C2 providers, the radar data became unreadable.

Quote on the Standard:
"The challenge for future C-UAS capabilities is... a distributed, resilient, and scalable C4ISR architecture that fuses multiple ISR sources. [Without this], agencies struggle to coordinate their activities... leading to the '17 screens' problem."
— Frequentis & NATO Lessons from Ukraine[1][2]

2. Examples of Vendor Lock-In (Aviation & Law Enforcement)

Below are specific examples where proprietary protocols or "walled garden" architectures created operational failures or severe dependencies.

A. The "Gatwick Incident" Response (Aviation/Law Enforcement)

Following the 2018 Gatwick drone sightings, the UK rushed to procure C-UAS equipment. The initial response relied on disjointed, proprietary systems that could not share data, leading to the infamous "swivel chair integration" where operators had to physically turn between different screens.

- The Lock-In: The airport initially deployed the AUDS (Anti-UAV Defence System), a bundled solution (Blighter Radar + Chess Dynamics Camera + ECS Jammer). While effective as a unit, it was a "black box." When the airport later wanted to add Metis Aerospace's Skyperion (RF sensor) to detecting drone pilots, it required complex, non-standard integration because the systems lacked a common language like SAPIENT.

- Result: This fragmentation forced the UK MoD to later mandate SAPIENT (Sensing for Asset Protection with Integrated Electronic Networked Technology) to prevent this scenario from repeating.

Quote:
"Even when drones can be successfully detected, incident handling is fragmented today... Automated risk analysis may lead to a decision to close the airport... but agencies struggle to coordinate their activities at speed and across large areas."
— Frequentis Whitepaper on Airport C-UAS[1]

B. DJI AeroScope (The "Walled Garden" Example)

Before the widespread adoption of Remote ID, DJI AeroScope was the only system that could reliably track DJI drones (90% of the market).

- The Lock-In: DJI encrypted the link between the drone and the AeroScope receiver. To view this data, law enforcement agencies were forced to use DJI’s proprietary software.

- The limitation: If a police force wanted to feed AeroScope data into a broader security system (e.g., to slew a PTZ camera from another vendor), they had to pay for a restricted, proprietary API. This made AeroScope a "data silo"—great on its own, but hostile to integration.

C. Early "Dedrone" Deployments (Correction Facility/Law Enforcement)

In its early iterations, Dedrone (now more open) operated as a "platform lock-in." They provided the DroneTracker software and encouraged users to buy Dedrone-branded RF sensors.

- The Lock-In: If a prison warden wanted to add a radar from a different manufacturer (e.g., Echodyne) to cover a blind spot, they often couldn't integrate it into the DroneTracker screen unless Dedrone had explicitly built a custom driver for it. This gave the vendor total control over which sensors the customer could upgrade to.

3. The "Cure": Standards to Reference

To avoid the lock-in described above, your project requirements should explicitly mandate compliance with these three standards. This matches the approach taken in the Denmark (DALO) project.

| Standard | Full Name | Why It Breaks Lock-In |
| --- | --- | --- |
| SAPIENT | Sensing for Asset Protection with Integrated Electronic Networked Technology | The "Silver Bullet": Developed by UK MoD/NATO. It forces every sensor to send data in a generic XML format (BSI Flex 335). It turns every sensor into a "plug-and-play" node. NATO adopted this specifically to stop vendor lock-in. |
| ASTERIX Cat 062/033 | Eurocontrol Surveillance Data Exchange | The Radar Fix: Unlike the older Cat 048, these newer categories are better suited for system tracking and can be standardized to avoid proprietary fields, provided the vendor adheres to the EUROCONTROL spec strictly. |
| MOSA | Modular Open Systems Approach | The US Mandate: A legal/acquisition strategy (Title 10 U.S.C. 4401) requiring US defense programs to use open interfaces. Referencing this forces US vendors to prove their system isn't a "black box." |

Recommendation:
When writing your requirements, use this phrasing to prevent the Gatwick/DJI scenario:

"The C-UAS Command & Control (C2) system must be vendor-agnostic and support SAPIENT (BSI Flex 335) standard for sensor ingestion. It shall not rely on proprietary ASTERIX User-Defined fields for essential drone classification data."

⁂

- https://www.frequentis.com/sites/default/files/support/2020-02/Frequentis-Counter-UAV-Whitepaper.pdf

- https://cepa.org/comprehensive-reports/an-urgent-matter-of-drones/

- https://www.gov.uk/guidance/sapient-autonomous-sensor-system

- https://www.unmannedairspace.info/wp-content/uploads/2022/06/Counter-UAS-directory.-May-2022.v3.pdf

- https://www.unmannedairspace.info/counter-uas-systems-and-policies/how-uk-government-and-airports-have-responded-to-gatwick-drone-chaos/

- https://www.faa.gov/sites/faa.gov/files/uas/resources/policy_library/uas_roadmap_2013.pdf

- https://www.easa.europa.eu/sites/default/files/dfu/amc_and_gm_to_implementing_regulation_eu_2021-664_-_issue_1_1.pdf

- https://www.bez-kabli.pl/news/technology-news-19-11-2025/

- https://www.science.gov/topicpages/i/integrating+software+architectures

- https://www.f6s.com/companies/iot/mo

- https://en.wikipedia.org/wiki/Gatwick_Airport_drone_incident

- https://digitalcommons.usf.edu/context/jss/article/2410/viewcontent/auto_convert.pdf

- https://iris.uniroma1.it/retrieve/c16d8e42-92a1-4767-b2a2-ba62f36a3784/Tesi_dottorato_Longo.pdf

- https://www.sciencedirect.com/topics/computer-science/network-discovery

- https://www.unmannedairspace.info/counter-uas-systems-and-policies/nato-to-adopt-uks-sapient-protocol-as-c-uas-standard/

- https://www.cto.mil/wp-content/uploads/2025/03/MOSA-Implementation-Guidebook-27Feb2025-Cleared.pdf

- https://cuashub.com/en/content/congress-seeks-dod-c-uas-assessments/

- https://tpcoe.gov.pl/download/10/4395/Protectionagainstunmannedaircraftsystems.pdf

- https://www.netiq.com/documentation/identity-manager-46/pdfdoc/setup/setup.pdf

- https://rosap.ntl.bts.gov/view/dot/73158/dot_73158_DS1.pdf