1) You don’t have one truth for “what is a track”

Crap moment: Operators see two drones on the map… it’s one drone. Or worse: one drone becomes “five” then disappears.

- Why it happens: Different vendors define “detection,” “plot,” “track,” “confidence,” and “drop” differently. Some radars output tracks that already include filtering. Some RF vendors output “detections” that look like tracks but are not stable. Remote ID is “self-reported track.”

- Mitigation: Define a strict internal lifecycle model: DETECTION → CANDIDATE_TRACK → CONFIRMED_TRACK → COASTING → DROPPED, and enforce it in fusion. Also define “source track” vs “fused track” and never mix them in the UI without labeling.

2) Time sync is “working”… until it isn’t, and fusion silently degrades

Crap moment: Everything looks normal, but correlation/fusion starts missing matches and confidence collapses—because one sensor drifted 30–300 ms and nobody noticed.

- Why it happens: PTP looks healthy at the grandmaster, but a boundary clock is misconfigured, a NIC falls back to software timestamping, a WAN segment introduces asymmetry, or a device goes into holdover badly.

- Mitigation: Treat time as a first-class health signal. Put “offset-from-master” and “time-validity” directly into the fusion logic (drop or down-weight late/bad-time sources). Alert on trend, not only threshold (“drifting faster than X µs/min”).

3) “Latency stacking” breaks fast-drone response, even with good protocols

Crap moment: Operators chase a ghost position—FPV drones move fast enough that your display is always behind, and intercept cues are late.

- Why it happens: Encoder buffers, AI inference queues, network jitter buffers, UI rendering—all add up.

- Mitigation: Make the COP show time-of-validity and predicted position (extrapolation) by default. Keep raw position (timestamped) available for forensics, but drive real-time cues off predicted state.

4) Multicast works in the lab, dies in production because of “one switch setting”

Crap moment: Video/radar floods the network or randomly disappears for some users.

- Why it happens: IGMP snooping quirks, querier placement, VLAN boundary behavior, incorrect PIM-SSM config, MLD issues on IPv6, or a firewall doing something “helpful.”

- Mitigation: Have a multicast acceptance test suite that runs in every site commissioning: join/leave latency, source filtering validation, querier failover, and “flood test.” Also maintain a “known good” network reference config for vendors.

5) Sensors lie—politely—and you don’t notice until an incident review

Crap moment: After an event, someone asks “how do you know that was a drone?” and you realize your system stored vendor “confidence=0.9” without knowing what it meant.

- Why it happens: Vendor confidence scores aren’t calibrated and often aren’t comparable across sensors.

- Mitigation: Normalize confidence into two layers:

  - Sensor-native confidence (verbatim, for debugging)

  - COP confidence (your calibrated score based on validation and historical performance)
And record the mapping version used.

6) “Coverage” overlays become legal/political landmines

Crap moment: Someone uses your coverage map as if it were gospel (“you said we cover this area”), then you miss something in that zone.

- Why it happens: Coverage depends on clutter, weather, drone RCS, line-of-sight, terrain, and configuration. Static circles on a map are almost always wrong.

- Mitigation: Represent coverage as probabilistic and conditional (“P(detect) vs range/altitude/class”) and show assumptions (weather mode, clutter model, minimum RCS). Also track coverage config versions and when they were last validated.

7) Cross-agency sharing fails on “policy latency,” not network latency

Crap moment: The system technically works, but in real ops: “I can’t see it” / “I can’t share it” / “legal says no.”

- Why it happens: ABAC is great, but if classification rules and downgrades aren’t agreed upfront, your default becomes “deny” or “overshare,” both bad.

- Mitigation: Define a data-sharing contract early:

  - what fields can be shared at each sensitivity tier

  - what obfuscation is acceptable (sensor location, exact timestamps, capability indicators)

  - what audit evidence is required
And automate downgrade transforms (don’t rely on humans during incidents).

8) Adapter/plugin ecosystem becomes unmaintainable without versioning discipline

Crap moment: A vendor firmware update changes one field; suddenly your ingestion silently mis-parses and fusion goes weird.

- Why it happens: Schema drift, undocumented changes, inconsistent unit conventions (meters vs feet), coordinate frames, and time formats.

- Mitigation: Require every adapter to declare:

  - interface version

  - units + coordinate frame

  - timestamp meaning (event time vs arrival time)

  - capabilities manifest
And put a contract test harness in CI that replays golden captures (PCAP + expected canonical outputs).

9) You underestimate “people and process” failure modes in the UI

Crap moment: Operators stop trusting the COP and revert to vendor UIs or phone calls.

- Why it happens: Too many alerts, unclear “why this is an alert,” or the UI hides uncertainty. Operators need explainability and calm.

- Mitigation: Build in:

  - alert fatigue controls (rate limiting, dedupe windows, escalation logic)

  - “why am I seeing this” explanations (source, confidence, corroboration)

  - explicit uncertainty cues (quality rings, stale indicators)
And have an ops-driven tuning cycle after go-live.

10) Incident replay and evidence handling are harder than real-time

Crap moment: A prosecutor/investigator asks for “all data about this event,” and you can’t reliably reconstruct what happened or what each user saw.

- Why it happens: You logged data, but not context: transforms, policy downgrades, UI subscriptions, and configuration state at the time.

- Mitigation: Treat replay as a first-class feature:

  - immutable event store with provenance

  - store policy decisions (“downgraded fields X,Y because rule Z”)

  - store config snapshots (fusion params, sensor configs, adapter versions)

  - store “user view” receipts for audit (“user A received track B at time T with fields masked”)

11) “Cyber” gets you through the front door via the supply chain, not the network

Crap moment: A sensor appliance has a hidden remote support tunnel or ships with outdated libraries; you discover it after deployment.

- Why it happens: Multi-vendor hardware/software supply chain is messy. Support teams want remote access. Default credentials happen.

- Mitigation: Procurement requirements should include:

  - SBOMs, vulnerability disclosure, patch SLAs

  - no undocumented outbound connectivity

  - disable remote support by default; break-glass process

  - signed firmware + secure boot where feasible
And do staged onboarding: quarantine VLAN → scan → certify → promote.

12) Remote ID becomes a “false sense of security”

Crap moment: Operators assume Remote ID = friendly/compliant, then get spoofed or see “valid-looking IDs” near an incident.

- Why it happens: Broadcast RID is easy to imitate; network RID might be unavailable; DRIP isn’t universal yet.

- Mitigation: Treat RID as one sensor with its own trust level. Display authentication status prominently:

  - Verified (DRIP ok)

  - Unverified (no DRIP)

  - Failed (bad auth)
And never allow RID alone to downgrade threat without corroboration.

13) Environmental reality: weather/clutter kills one modality; you need graceful degradation

Crap moment: Fog kills EO, rain/clutter hurts some radars, RF gets multipath—operators think the system is broken.

- Why it happens: Every modality has failure modes, often correlated with the same conditions.

- Mitigation: Design “degraded mode” UX and logic:

  - show which modalities are impaired (“EO visibility low”, “RF noise high”)

  - automatically change fusion weighting

  - have SOPs that switch to alternate sensors and change thresholds

14) You’ll get blamed for misses you couldn’t detect—unless you record assumptions

Crap moment: Someone asks “why didn’t we detect this?” and you can’t prove the system wasn’t capable under that scenario.

- Why it happens: No stored operational envelope: min detectable RCS, altitude limits, sensor masks, downtime windows.

- Mitigation: Record “capability state” continuously:

  - sensor health and mode

  - coverage model version

  - active masks/blocked sectors

  - weather mode parameters (if used)
So you can explain misses credibly.

If you only fix three things early

- Contract tests + replay harness for adapters (prevents silent breakage).

- Time-validity everywhere (drop/down-weight bad timestamps; show staleness).

- Evidence-grade provenance (what happened, what was transformed, who saw what).

Modular multi-vendor sUAS detection common-operating-picture: Standards and protocols

This briefing summarises existing open standards and widely adopted protocols that can underpin a modular “adapter/plugin” ecosystem for a national multi-agency drone/sUAS detection common operating picture (COP). The goal is to combine radar, RF, remote-ID, acoustic and electro-optical (EO) sensors from multiple vendors into a unified map-based interface and to enforce role-based access to detections and historical tracks. Where possible, the report leans toward open standards used in Modular Open Systems Architecture (MOSA) and Sensor Open Systems Architecture (SOSA), along with protocols already adopted by Anduril’s Lattice platform (open integration of third-party sensors)unmannedairspace.info. Citations are provided to give authoritative references for each standard.

1. System architecture baseline

1.1 Network and multicast

| Area | Standards & recommendations | Why they matter |
| --- | --- | --- |
| IP segmentation & multicast | Source-specific multicast (SSM) from RFC 4607 designates IPv4 addresses 232.0.0.0/8 and IPv6 prefix FF3x::/32 for SSM groupsrfc-editor.org. IGMPv3 and MLDv2 allow hosts to inform routers about their desire to receive multicast traffic and support filtering by sourcerfc-editor.org. Use PIM-SSM routing with IGMPv3 (IPv4) or MLDv2 (IPv6) to ensure receivers request traffic only from specific sensor sources; avoid any-source multicast to limit cross-deliveryrfc-editor.org. | Segregating multicast traffic by vendor and sensor type reduces noise and simplifies access control. SSM ensures an (S,G) channel for each sensor feed and eliminates the need for rendezvous points, making it attractive for real-time video and radar streams. |
| Campus TSN & QoS | Time-Sensitive Networking (TSN) set in IEEE 802.1 standards: 802.1Qbv (Time-Aware Shaper) schedules deterministic transmission, 802.1Qbu/802.3br enable frame pre-emption to reduce guard-band delays, 802.1CB adds Frame Replication and Elimination for reliability, and 802.1Qci provides per-stream filtering and policingwindriver.com. TSN leverages 802.1AS (gPTP) for precise clock synchronisation, and other resource-reservation standards like 802.1Qav (credit-based shaper) now consolidated in 802.1Q-2022windriver.com. | TSN ensures bounded latency and zero-congestion loss for sensor video, radar and control traffic. Using TSN with VLAN segmentation and per-flow scheduling allows mixing deterministic sensor streams with best-effort IT traffic on the same network without interference. |
| WAN backhaul & resilience | For WAN or backhaul links, leverage open streaming protocols such as Reliable Internet Stream Transport (RIST) or Secure Reliable Transport (SRT). RIST main profile defines levels: baseline features (GRE tunnelling, periodic keep-alives), optional features such as null-packet deletion and tunnel bondingstatic.vsf.tv. The DTLS level provides end-to-end encryption via Datagram TLS and supports server- and client-side certificatesstatic.vsf.tv; the PSK level supports pre-shared keys and key rotationstatic.vsf.tv. SRT is an emerging IETF draft that provides reliability and security over UDP; it introduces control packets, improved flow/congestion control and optional encryption for low-latency streaminghaivision.github.io. | RIST and SRT are open specifications widely adopted in broadcast. They combine adaptive retransmission (ARQ) and optional FEC for robust streaming over lossy networks. Choosing open protocols avoids vendor lock-in; RIST has explicit profiles for PSK and DTLS encryption, which are attractive for national-level deployments, while SRT is an active IETF work with similar goals. |
| Backhaul encryption & port admission | MACsec (IEEE 802.1AE) provides line-rate encryption and integrity at the Ethernet layer; it uses AES-GCM and relies on 802.1X/EAP for key exchange. MACsec protects frames from eavesdropping and tampering and supports hardware-accelerated encryptionfs.com; it can be combined with 802.1AR device identities and 802.1X port access control, which requires clients to authenticate before receiving network accessauvik.com. | Because sensors will be deployed across agency networks, link-layer security ensures confidentiality and integrity even if higher-layer security is disabled. Port-based network access prevents rogue devices from injecting false tracks. |

1.2 Precision timing and synchronisation

| Area | Standards & recommendations | Why they matter |
| --- | --- | --- |
| Precision Time Protocol (PTP) | IEEE 1588 (PTP) synchronises clocks across devices to the sub-microsecond level. PTP uses a hierarchical structure (grandmaster, boundary/transparent clocks), exchange of sync and delay messages, and precise time stampinghpe.com. Profiles tailored for telecommunications, industrial automation and AI exist. | With multiple radars and RF sensors, accurate time stamping is vital to fuse tracks and determine velocities. Deploy grandmasters and boundary clocks across networks; evaluate PTP High-Accuracy or White Rabbit for sub-100-ns triggers if required. |
| PTP security | IEEE 1588d extends PTP with security features (Prong A AUTH TLV, GDOI key distribution). MACsec or TLS at the transport layer (Prong B) and architectural hardening (Prongs C & D) remain mandatory for critical infrastructures. | Prevents spoofing or delay attacks that could corrupt track fusion. Use PTP AUTH TLV when vendor support is available; otherwise, rely on MACsec to protect PTP packets. |
| gPTP/802.1AS for TSN | For layer-2 TSN networks, gPTP/802.1AS provides synchronisation with nanosecond-level accuracy and reduces packet delay variationwindriver.com. | gPTP ensures determinism required by TSN scheduling and can integrate with 1588 boundary clocks over WAN. |

1.3 Logging, telemetry and configuration

| Area | Standards & recommendations | Why they matter |
| --- | --- | --- |
| Syslog and secure transport | RFC 5424 defines the modern syslog message format with structured data and vendor-specific extensionsdatatracker.ietf.org. RFC 5425 specifies transporting syslog over TLS, describing how TLS secures syslog messages and mitigates threats such as eavesdroppingdatatracker.ietf.org. RFC 6587 defines framing for syslog over TCP. Use default port 6514 for syslog-TLS and restrict cipher suites per RFC 9662. | Collect sensor logs, detection events and auditing records in near real-time. Secure transport ensures logs remain confidential and tamper-proof across agency boundaries. |
| Model-driven telemetry | NETCONF (RFC 6241) provides secure, transaction-oriented configuration management over SSH/TLS, supports multiple datastores and uses YANG modelscodilime.com. RESTCONF (RFC 8040) offers a RESTful API to access YANG data via HTTP methods, using JSON or XMLcodilime.com. gNMI (gRPC Network Management Interface) from OpenConfig uses gRPC/HTTP2 and protocol-buffers and supports streaming telemetry and atomic transactionscodilime.com. | Standardising configuration and telemetry across heterogeneous sensors simplifies integration and auditing. YANG models can define common capabilities (e.g., track output rate, antenna orientation) and allow adapters to map vendor-specific parameters to a common schema. |

1.4 Quality of Service (QoS)

Quality of Service is critical to ensure that control messages and high-rate sensor data do not impact each other. DSCP values in IP headers request different per-hop behaviors. Cisco’s documentation lists Expedited Forwarding (EF) (DSCP 46) for high-priority low-latency traffic, Voice-Admit (DSCP 44) for voice-controlled flows, and Assured Forwarding (AF) classes (e.g., AF41, DSCP 34) for video streamsdocs.thousandeyes.comdocs.thousandeyes.com. The NQB draft defines DSCP 45 for non-queue-building flows that need low latency but minimal bandwidth, recommending that network devices queue DSCP 45 traffic separatelyietf.org. Apply DSCP markings consistently across sensors and networks and map them to TSN schedules or priority queues.

2. Sensor interfaces and data standards

2.1 Radar

- EUROCONTROL ASTERIX is a widely used set of open interface definitions for aviation surveillance. The ASTERIX library contains over seventy message categories; Category 062 (CAT-062) provides system track data and is used for non-cooperative radar tracksrosap.ntl.bts.gov. Category 033 carries ADS-B messages, and Category 129 can encode Remote-ID informationrosap.ntl.bts.gov. ASTERIX data formats include exact bit-level definitions, making them interoperable across vendors. Use CAT-062 for radars and GMTI sensors, ensuring that packet sizes avoid IP fragmentation.

- STANAG 4607 (GMTI) is a NATO standard for Ground Moving Target Indicator data; it defines a flexible message format that can be tailored to send detailed data for targeting or minimal data for situational awareness, promoting interoperabilitymitre.org.

- STANAG 4609 is the NATO digital motion imagery standard for Full-Motion Video. It encapsulates H.264/H.265 video in MPEG-TS with MISB Key-Length-Value (KLV) metadata (e.g., platform position, sensor orientation, timestamps)impleotv.com. Use MISB ST 0601/0603/0604 for KLV metadata and ST 0903 for Video Moving Target Indicator (VMTI) metadataimpleotv.com. Ensuring adherence to STANAG 4609 allows video streams to interoperate across coalition partners.

- STANAG 4676 defines the data model and message formats for exchanging tracks among ISR systems; it aims to promote interoperability in the production, exchange and exploitation of tracking datanisp.nw3.dk. Use STANAG 4676 for inter-system track exchange between the COP and external defence/aviation networks.

2.2 RF and signal intelligence

- ANSI/VITA 49.2 (VRT) defines VITA Radio Transport for radio frequency (RF) signal transport. It structures sample payloads with context packets that carry metadata such as bandwidth, centre frequency and timestamps. Adopt VRT for SDR front ends and ensure timestamping is synchronised via PTP.

- SigMF (Signal Metadata Format) provides an open standard for recording and sharing RF signal captures. It separates binary I/Q data from JSON metadata, making it easier to share datasets across agencies. Use SigMF for archival and forensic RF data.

2.3 Electro-optical/Infra-red (EO/IR) and acoustic

- STANAG 4609 and MISB standards apply to EO/IR video streams (see above). For audio and acoustic sensors, use AES67 for interoperability; AES67 interoperates with PTP-synchronised audio transport and ties into SMPTE ST 2059-2 timing guidelines. Ensure PTP parameters match across AES67 and SMPTE profiles to maintain lip-sync and cross-sensor timing alignment.

2.4 Remote-ID

- ASTM F3411-22a defines UAS Broadcast and Network Remote-ID. The broadcast message includes unique identifiers, location, altitude, speed and operator information. The standard is widely adopted by regulators (FAA/ EASA). Sensors should ingest ASTM broadcast messages and network Remote-ID messages to populate the COP.

- IETF DRIP adds trust to remote-ID. RFC 9374 introduces Hierarchical Host Identity Tags (HHITs) – self-asserting IPv6 addresses providing trustable identifiers with explicit hierarchy to facilitate registry discoveryrfc-editor.org. RFC 9575 defines authentication formats that allow observers to verify that broadcast Remote-ID messages are signed by the registered owner of the devicerfc-editor.org. Implement DRIP validation in the ingestion pipeline to accept only remote-ID messages with valid signatures and accepted registries.

3. Security and access control

- Network admission – Deploy 802.1X for port-based access control so that only authenticated sensors join the network. 802.1X uses AAA servers (often RADIUS) to authenticate clients and can provide authorization and accountingauvik.com. Use MACsec for per-hop link encryption to ensure confidentiality and integrity across switchesfs.com.

- Transport security – For control channels (NETCONF/gNMI) and syslog, use TLS 1.3 (RFC 8446) with mutual authentication. For streaming data, select RIST DTLS level or PSK level for encryptionstatic.vsf.tvstatic.vsf.tv. Avoid legacy protocols such as SSL or IPsec unless mandated.

- Role-based visibility – The COP should enforce attribute-based access control. Role definitions can be maintained in an identity management system; sensor outputs can be tagged with sensitivity levels. For example, defence sensors may mask precise coordinates when displayed to civilian operators while still providing aggregated coverage zones.

- Audit and logging – Use RFC 5424 syslog with TLS transport for every action on the COP. Keep audit logs for at least the regulatory retention period. Structured data elements (e.g., user ID, action, track ID) enable post-incident investigations. Consider using Message Bus events (e.g., AMQP 1.0 or MQTT 5) with persistent topics for streaming logs.

4. Modular integration (“adapter/plugin” model)

A modular COP should accept sensor feeds through adapters that translate vendor-specific formats into a normalised track/event schema. Design guidelines:

- Define a canonical data model for detections and tracks. Use STANAG 4676 or a custom YANG-derived schema for the canonical model. Adapters map vendor fields (e.g., radar SNR, RF frequency) into this schema and drop unsupported fields. This decouples the core system from vendor updates.

- Use open transport protocols and message buses. Consider DDS/RTPS for real-time publish/subscribe within the mission network; define topics for tracks, video frames and alerts with quality-of-service parameters matching DSCP and TSN priorities. For web-based clients, deliver summarised data via web sockets or gRPC, enforcing the same access-control rules.

- Implement dynamic plugin registration. Each adapter should declare its capabilities via a YANG or JSON manifest (supported message categories, update rate, authentication requirements). When a new sensor is added, its adapter is deployed and registered with the COP; the system automatically subscribes to its (S,G) multicast channels and maps its outputs to the canonical model.

- Test interoperability. Use conformance tests: ensure ASTERIX frames adhere to the specified edition (e.g., CAT-062 Ed 1.21), verify PTP timing accuracy, TSN schedule coherence and RIST encryption profiles. Accept only sensors that pass baseline and security tests.

- Support scalability. Use load-balanced ingest clusters to handle bursts of detections, replicate data using 802.1CB and RIST bonding for redundancy, and implement distributed caching for historical track queries.

5. Adoption considerations and pitfalls

Benefits of open standards

- Interoperability and vendor diversity – SOSA and MOSA frameworks emphasise open standards to permit rapid integration of new sensors and effectors. Anduril’s Lattice platform demonstrates this by integrating third-party sensors through an open architectureunmannedairspace.info. Aligning with widely adopted standards (ASTERIX, STANAG 4676, IEEE 802.1Qbv) reduces integration effort and fosters a competitive vendor market.

- Future-proofing – Standards such as TSN and PTP are actively evolving; aligning early ensures compatibility with next-generation sensors (e.g., high-bandwidth phased-array radars). RIST and SRT are community-driven and likely to remain relevant in broadcast and defence streaming.

- Security – Adhering to MACsec, TLS, 802.1X and DRIP provides defence-in-depth. Standardised logging and telemetry facilitate audits.

Potential pitfalls and alternative perspectives

- Standards divergence – Many standards exist with overlapping scope (e.g., ASTERIX vs. STANAG 4676 vs. custom vendor formats). Selecting one canonical format may alienate sensors that lack support. To mitigate, implement flexible adapters and support multiple input formats while converging on a unified internal schema.

- Latency vs. reliability trade-offs – TSN and RIST provide deterministic timing and reliability but increase complexity and may require expensive switches. In some deployments, simple QoS with DSCP and DiffServ may suffice. Evaluate the cost/benefit before adopting full TSN across the network.

- Regulatory changes – Remote-ID standards and UAS regulations are evolving. Investing heavily in ASTM F3411-22a and DRIP now may necessitate updates when regulators adopt new frameworks. Consider modular remote-ID ingest to adapt to future protocols.

- Vendor lock-in disguised as open – Even with open standards, some vendors may implement proprietary extensions or licensing terms. Conduct thorough interoperability tests and demand open documentation to avoid hidden lock-in.

- Complexity of security management – Implementing MACsec, 802.1X, TSN and DRIP concurrently demands careful key management and configuration. Automation (e.g., using NETCONF and gNMI) can help but also introduces new attack surfaces; maintain rigorous security hygiene and auditing.

By embracing the above standards and protocols, the multi-vendor sUAS detection COP can achieve a modular, vendor-agnostic architecture. Using open, widely adopted standards reduces integration risks and enables rapid onboarding of new sensors while maintaining security, deterministic performance and auditability.

Below is a market-survey of available sensors and systems that already implement the open standards and protocols recommended in the report. These examples show that the standards—ASTERIX, STANAG 4609, SAPIENT, Remote-ID, TSN/PTP, etc.—are supported by multiple vendors, giving procurement teams a choice of compatible equipment.

Radar sensors

| Vendor/product | Evidence of standards support | Notes |
| --- | --- | --- |
| Leonardo DRS RADA nMHR & ieMHR radars | The nMHR radar’s specification lists “Interface protocols: ASTERIX, customer-tailored” over Ethernetdrsrada.com; the ieMHR radar lists the samedrsrada.com. | Multi-mission X-band radars used for counter-UAS and air-defence; their ASTERIX interface simplifies integration into a heterogeneous C2 environment. |
| Blighter A800 Mk-2 multi-mode radar | The datasheet states that the radar interfaces via the BlighterNexus hub and can output to industry-standard C2 formats including XML, ASTERIX and SAPIENT, and that an SDK is available for developersblighter.com. | Supports ASTERIX tracks and the UK MOD SAPIENT standard, allowing plug-in integration. |
| Robin Radar IRIS | The product description notes that the IRIS drone-detection radar can be controlled using Asterix, SAPIENT and a proprietary XML-APIeverythingrf.com. | Demonstrates European radar vendor support for ASTERIX and SAPIENT. |
| Navtech Radar | Navtech’s documentation (not publicly viewable as text) states that its radars can output radar video in ASTERIX CAT-240 format (indicated on their support pages). | Provides open radar video to multi-vendor systems. |
| Echodyne EchoGuard/EchoShield | A Defense Advancement article on Echodyne mentions that the company provides an API and can translate radar data to widely used industry standards; radar data is sometimes transmitted using ASTERIX, while the UK uses the SAPIENT interfacedefenseadvancement.com. | Shows that emerging AESA radars can supply standard outputs. |
| HENSOLDT Xpeller | News coverage highlights that the Xpeller counter-UAS system has a modular, open-interface architecture allowing additional sensors to be integratedsourcesecurity.com. | Not explicit about ASTERIX but indicates an open, plug-in-friendly design. |
| Cambridge Pixel radar interface software | Cambridge Pixel’s VSD and RadarWatch products accept open data formats such as ASTERIX, SAPIENT and NMEA-0183 and integrate a wide range of radars and camerascambridgepixel.comcambridgepixel.com. | Useful when fusing heterogeneous sensors; they support plug-in inputs and provide ASTERIX/SAPIENT outputs. |

RF detectors and RF-based trackers

| Vendor/product | Standards support | Notes |
| --- | --- | --- |
| CRFS RFeye Nodes/Software | The RFeye software suite exports I/Q recordings via open formats and can stream Time-Difference-of-Arrival geolocations using VITA-49, ASTERIX and TCP/IP; its APIs use REST methods and JSONcrfs.comcrfs.com. The Node Plus 100-18 sensor’s specification notes that geolocation streaming supports “standard ASTERIX and Google Protocol Buffer data formats”crfs.com. | Enables integration of RF detection (direction-finding and geolocation) into an ASTERIX-based track fusion engine. |
| Fortem SkyDome | Fortem’s C2 platform integrates radar, cameras and RF sensors and can connect to third-party C2 systems via a rich APIfortemtech.com. | While Fortem does not publicly state ASTERIX support, the API approach allows a plug-in adapter to feed normalized data into the COP. |

Remote-ID receivers and network RID

| Vendor/product | Standards support | Notes |
| --- | --- | --- |
| Dronetag Rider / Dronetag Scout | The Rider product advertises compliance with ASTM F3411-22A and EN 4709-002 remote-ID standardsdronetag.com. The Scout remote-ID receiver is open-source and transmits received Remote-ID information in JSON via MQTT while using the Open Drone ID framework that implements ASTM F3411-22a-RID-Bdronescout.co. | Suitable for network remote-ID sensors; open interfaces enable integration. |
| uAvionix pingRID | The pingRID transmitter meets FAA 14 CFR Part 89 and ASTM F3411-22a remote-ID requirementsuavionix.comuavionix.com. | Useful for equipping friendly drones; data can be consumed via network remote-ID interfaces. |
| DroneScout ds230/ds240 receivers | The technical manual states the receivers comply with ASTM F3411-22a-RID-B and DIN EN 4709-002 and provide broadcast RID scanning via Bluetooth/Wi-Fi (citation from previous research). | Provides a sensor that can feed broadcast remote-ID into the COP via an adapter. |

EO/IR cameras and encoders with STANAG 4609 metadata

| Device | Standards support | Notes |
| --- | --- | --- |
| Atlas UAS payloads (Visor FF, KALOS 120, Argus M20) | These camera payloads list STANAG 4609 as the live-stream metadata format and for recorded videoatlasuas.comatlasuas.comatlasuas.com. | Provides MISB/IMC-compliant video for track fusion and evidence. |
| Teledyne FLIR EO/IR Mk-II | Features note “embedded geotags and STANAG 4609-compliant metadata” enabling precise geolocationdefense.flir.com. |  |
| FLIR TacFLIR 380HD | The spec states metadata is MISP-compliant and lists STANAG 4609 among supported metadata standardsbeechwoodequipment.com. |  |
| FLIR HDZoom 30 | Product page notes STANAG 4609 metadatafarrwest.com. |  |
| Z3 Technology CamL-15 encoder | This dual-camera encoder supports H.265/H.264 streaming, SRT/RTMP protocols and advertises compliance with STANAG 4609 and MISB 0601/0604 metadata; it can encrypt streams and uses SRT for reliable transportaegis-elec.comaegis-elec.com. | Useful for adding standards-compliant metadata to other sensor feeds. |

Networking equipment supporting TSN, PTP and security

| Product | Standards support | Notes |
| --- | --- | --- |
| Milpower MILTECH 9117 TSN switch | The rugged switch supports Time-Aware Shaper (IEEE 802.1Qbv), frame pre-emption (802.1Qbu) and per-stream filtering (802.1Qci) for deterministic TSN deliverymilpower.com. It provides hardware timestamping and synchronization compliant with IEEE 802.1AS and IEEE 802.1Qbv, and supports PTP with boundary clocks, SyncE, MACsec encryption and 802.1X authenticationmilpower.commilpower.commilpower.com. | Demonstrates that network hardware exists today to deliver sub-microsecond synchronization, TSN quality-of-service, and link-layer security, aligning with the architecture’s requirements. |

C2 / display software

| Product | Standards support | Notes |
| --- | --- | --- |
| Cambridge Pixel VSD / RadarWatch | The VSD and RadarWatch software accept open formats including ASTERIX and SAPIENT and integrate radars (e.g., Blighter, Echodyne, HENSOLDT Spexer, RADA), cameras and ADS-B receiverscambridgepixel.comcambridgepixel.com. | Provides an off-the-shelf common operating picture that can fuse multiple sensor modalities. |
| Fortem SkyDome Manager | Fortem’s manager integrates radar, cameras, RF sensors and mitigation systems and can interface with third-party C2 via APIfortemtech.com. | While not explicitly using ASTERIX, its API approach allows integration via adapters. |

Assessment and potential pitfalls

- Availability – Numerous radar, RF and camera vendors already support ASTERIX (for track/video), SAPIENT (UK C-UAS standard), STANAG 4609/MISB for video metadata, and ASTM F3411 for Remote-ID. This demonstrates that adopting these standards in procurement specifications will not limit choice; multiple vendors can meet the requirements.

- Integration effort – Many products provide proprietary APIs alongside standard outputs. Integrators should verify that ASTERIX/SAPIENT support covers all required fields (e.g., altitude, track status) and that metadata timing aligns with PTP. Some sensors may require custom adapters to convert proprietary messages into standardized formats.

- Evolving standards – Remote-ID trust mechanisms (IETF DRIP), TSN security (NTS4PTP) and certain IETF drafts (NQB) are still evolving. Procurers should allow for updates as these standards mature.

- Interoperability testing – Even when products claim ASTERIX or STANAG compliance, test interoperability early. Variations in optional fields or vendor extensions may require negotiation.

Overall, the market offers a variety of sensors and systems that support the open standards outlined in the architecture. Using ASTERIX, STANAG 4609, SAPIENT, and remote-ID standards in procurement specifications should therefore facilitate multi-vendor integration rather than restrict supplier choice.

Review of Standards and Protocols in Light of Market-Available Systems

Context

The planned multi-vendor common operating picture (COP) architecture emphasises modular, open standards and interoperability (MOSA/SOSA) to fuse detections and tracks from radar, RF, electro-optical/infra-red (EO/IR), acoustic and Remote-ID sensors. Key elements include:

- Sensor interfaces: ASTERIX CAT-062/240 for radar tracks/video; STANAG 4607 for GMTI; STANAG 4609 + MISB metadata for full-motion video; SAPIENT for multi-modal track exchange; ANSI/VITA 49.2 and SigMF for RF; ASTM F3411 (broadcast & network Remote-ID) with DRIP authentication.

- Network: VLAN-segmented IP fabric; source-specific multicast using PIM-SSM with IGMPv3/MLDv2; TSN and Precision Time Protocol (PTP) for deterministic latency; 802.1X and MACsec for link security.

- Backhaul/streaming: RIST or SRT for WAN resilience; 802.1CB (frame replication) for hitless failover.

- Telemetry/logging: NETCONF/RESTCONF/gNMI for configuration and telemetry; RFC 5424 syslog over TLS; DSCP mapping (EF, AF41, NQB) for QoS.

The user requested a review of this plan based on the actual sensors and systems on the market, and asked to consider Anduril’s Lattice standards and protocols as inspiration.

Evidence from available systems

Radar

- Many radar vendors support ASTERIX output. RADA’s nMHR and ieMHR radars offer Ethernet interfaces with ASTERIX protocolsdrsrada.comdrsrada.com. Blighter’s A800 Mk-2 radar outputs XML, ASTERIX and SAPIENT and provides an SDKblighter.com. Robin Radar’s IRIS can be controlled using ASTERIX, SAPIENT or a proprietary XML-APIeverythingrf.com. Cambridge Pixel’s VSD/RadarWatch software accepts ASTERIX and SAPIENT and integrates numerous radars and EO/IR sensorscambridgepixel.comcambridgepixel.com. These examples demonstrate that ASTERIX and SAPIENT are widely implemented in radar and display systems.

RF sensors

- CRFS RFeye sensors and software can stream geolocation data via VITA-49, ASTERIX and TCP/IP, with RESTful JSON APIscrfs.comcrfs.comcrfs.com. This means that specifying ASTERIX and open APIs for RF geolocators is realistic.

EO/IR and video

- Commercial payloads support STANAG 4609 metadata. Atlas UAS payloads (Visor FF, KALOS 120, Argus M20) list live-stream and recording metadata as STANAG 4609atlasuas.comatlasuas.comatlasuas.com. FLIR’s EO/IR Mk-II and TacFLIR 380HD cameras embed STANAG 4609-compliant metadatadefense.flir.combeechwoodequipment.com. The FLIR HDZoom 30 also lists STANAG 4609 metadatafarrwest.com. The Z3 CamL-15 dual-camera encoder transmits SRT/RTSP streams and is STANAG 4609/MISB 0601/0604 compliantaegis-elec.comaegis-elec.com. Thus, using STANAG 4609 with MISB metadata for video is realistic.

Remote-ID

- Dronetag Rider/Scout Remote-ID receivers comply with ASTM F3411-22A and EN 4709-002 and provide REST API/MQTT outputsdronetag.comdronescout.co. uAvionix pingRID transmitters meet FAA Part 89 and ASTM F3411-22auavionix.comuavionix.com. Commercial receivers like DroneScout ds230/ds240 also use ASTM F3411-22a and open frameworks. However, the DRIP trust standard (RFC 9575) is very new; no COTS receiver currently advertises DRIP message verification. Our plan should include DRIP for future growth but not require it in initial procurements.

TSN/PTP and secure networking

- Time-sensitive networking equipment is available. Milpower’s MILTECH 9117 switch supports TSN features (802.1Qbv time-aware shaper, 802.1Qbu frame pre-emption, 802.1Qci filtering), hardware timestamping with IEEE 802.1AS and PTP, SyncE, MACsec and 802.1Xmilpower.commilpower.commilpower.commilpower.com. While many sensors use standard Ethernet rather than TSN, TSN-capable switches ensure deterministic transport and sub-microsecond synchronisation for fusion; adopting PTP in the network is therefore practical.

Backhaul streaming

- Video encoders such as Z3 CamL-15 support SRT streaming with encryptionaegis-elec.comaegis-elec.com. While RIST is advocated for WAN multicast resilience, off-the-shelf sensors rarely implement it. Backhaul gateways (e.g., from broadcast/video industry) may support RIST; thus specifying RIST or SRT as acceptable options provides flexibility.

Logging, telemetry and QoS

- Standardised logging and telemetry protocols (RFC 5424 syslog, NETCONF/RESTCONF/gNMI, DSCP code points) are widely implemented in network and compute equipment. Many sensors provide REST APIs or JSON outputs (e.g., CRFS, Dronetag) that can be instrumented via these protocols.

Anduril Lattice concepts

- Anduril’s Lattice OS uses an open architecture and modular microservices design. A product cheatsheet summarises that Lattice is “designed with an open architecture for seamless JADC2 integration, adhering to standards like OMS/UCI, FACE and MOSA”cheatsheets.davidveksler.com and that its microservices allow rapid integration of new hardware and sensorscheatsheets.davidveksler.com. The Lattice SDK provides gRPC/HTTP APIs and open data models, enabling third parties to integrate sensors and effectorscheatsheets.davidveksler.com. A Breaking Defense article notes that Lattice Mesh allows frontline units to subscribe to data feeds from heterogeneous sensors and that Anduril is opening the SDK so third-party developers can write “translator” modules for additional sensorsbreakingdefense.com. These references show that Anduril’s approach aligns with MOSA principles: open APIs, multiple data formats, and developer kits enabling vendor-agnostic integration.

Recommendations and adjustments

- Stick with ASTERIX for radar tracks and VITA-49/SigMF for RF. Many radars and RF sensors already output ASTERIX; open C2 software (e.g., Cambridge Pixel) accepts it. For radar video, use ASTERIX Cat 240; for GMTI, adopt STANAG 4607. Maintaining ASTERIX also aligns with SAPIENT, which is essentially an ASTERIX-compatible open standard.

- Maintain STANAG 4609 + MISB metadata for video, but allow SRT or RIST transport. EO/IR sensors widely support 4609. SRT is more commonly implemented than RIST in video encoders; specify either SRT or RIST for WAN backhaul to ensure equipment choice.

- Retain SAPIENT as an optional multi-modal interface. While not universally adopted, SAPIENT is supported by several radar vendors (Blighter, Robin), Cambridge Pixel software, and the UK MoD. Adopting it can simplify future integration. However, ASTERIX remains more widely supported; we should accept either ASTERIX or SAPIENT for track exchange.

- Support ASTM F3411 Remote-ID; treat DRIP as future growth. Remote-ID receivers already comply with ASTM F3411 and broadcast their data via open APIs. DRIP (RFC 9575) should be included in the architecture for future authentication, but procurement specs should not require DRIP until COTS receivers implement it.

- Adopt PTP across the network and specify TSN where needed. TSN-enabled switches such as MILTECH 9117 are available, but most sensors have basic Ethernet. Use PTP (IEEE 1588-2019) to timestamp data at the sensor and maintain time coherence; deploy TSN on campus networks carrying high-bandwidth radar or video. Use MACsec and 802.1X for link security.

- Use RIST or SRT for resilient streaming, with vendor choice. Because only some encoders support RIST, allow SRT as an alternative. Specify that WAN gateways MUST support at least one of these protocols and optional 802.1CB frame replication for hitless failover.

- Leverage open APIs and microservices similar to Anduril Lattice. Following Anduril’s example, provide a well-documented SDK with gRPC/REST endpoints and open data models (for entities, tasks and objects). Encourage vendors to write adapters/translators to this API, similar to Lattice’s translator conceptbreakingdefense.com. Where appropriate, align with open standards like OMS/UCI or FACE (used by Lattice OS) to broaden the ecosystemcheatsheets.davidveksler.com.

- Maintain MOSA principles: multiple options for each function. For example, permit either ASTERIX or SAPIENT for tracks; RIST or SRT for streaming; PTP or gPTP for timing. This reduces vendor lock-in and allows integration of both legacy and cutting-edge equipment.

Potential pitfalls

- Incomplete adoption of SAPIENT and DRIP. While they are promising, only a subset of vendors support SAPIENT, and DRIP implementations are nascent. Mandating them could limit vendor choices; treat them as optional or future requirements.

- Complex integration and testing. Variations in ASTERIX implementations, metadata fields or DSCP mappings may require adapter development and thorough validation. A robust SDK and certification process (similar to Anduril’s Lattice partner program) will be necessary.

- TSN cost and maturity. TSN switches and NICs are available but increase cost and complexity; not all sensors support TSN. Consider TSN for high-demand segments (e.g., radar video) and use standard Ethernet elsewhere.

Conclusion

The market already offers radar, RF, EO/IR and Remote-ID devices that conform to the key standards in the proposed architecture. The planned protocols therefore provide a solid foundation for a multi-vendor C-UAS COP. However, flexibility is important: offering options (ASTERIX or SAPIENT; SRT or RIST; PTP or gPTP) will accommodate more vendors and ease integration. Borrowing from Anduril’s Lattice approach—open APIs, microservices and developer kits—can further encourage a plug-in ecosystem and avoid vendor lock-in.